classdef LimitedJointVelocityController < handle

properties

robot

stiffness

end

methods

function self = LimitedJointVelocityController(stiffness, robot)

self.stiffness = stiffness;

self.robot = robot;

end

function controlAcceleration = computeCommand(self, desired_velocity, q_dot, full_jac)

% Clamping max velocity

if norm(desired_velocity) > 5 %norm: tính độ dài sqrt

desired_velocity = desired_velocity / norm(desired_velocity) * 5;

end

% Computing joint velocity from cartesian velocity

full_jac_inv = full_jac' / (full_jac * full_jac' + 0.01 * eye(6));

q_dot_des = (full_jac_inv * [0; 0; 0; desired_velocity]);

% Controlled acceleration from desired joint speed

controlAcceleration = self.stiffness * (q_dot_des - q_dot);

end

end

end