function [x xd t xT x_obs]=Simulation(x0,xT,fn_handle,varargin)

%

% This function simulates motion that were learnt using SEDS, which defines

% a motins as a nonlinear time-independent asymptotically stable dynamical

% systems:

%                               xd=f(x)

%

% where x is an arbitrary d dimensional variable, and xd is its first time

% derivative.

%

% The function can be called using:

%       [x xd t]=Simulation(x0,xT,Priors,Mu,Sigma)

%

% or

%       [x xd t]=Simulation(x0,xT,Priors,Mu,Sigma,options)

%

% to also send a structure of desired options.

%

% Inputs -----------------------------------------------------------------

%   o x:       d x N matrix vector representing N different starting point(s)

%   o xT:      d x 1 Column vector representing the target point

%   o fn_handle:  A handle function that only gets as input a d x N matrix,

%                 and returns the output matrix of the same dimension. Note

%                 that the output variable is the first time derivative of

%                 the input variable.

%

%   o options: A structure to set the optional parameters of the simulator.

%              The following parameters can be set in the options:

%       - .dt:      integration time step [default: 0.02]

%       - .i_max:   maximum number of iteration for simulator [default: i_max=1000]

%       - .plot     setting simulation graphic on (true) or off (false) [default: true]

%       - .tol:     A positive scalar defining the threshold to stop the

%                   simulator. If the motions velocity becomes less than

%                   tol, then simulation stops [default: 0.001]

%       - .perturbation: a structure to apply pertorbations to the robot.

%                        This variable has the following subvariables:

%       - .perturbation.type: A string defining the type of perturbations.

%                             The acceptable values are:

%                             'tdp' : target discrete perturbation

%                             'tcp' : target continuous perturbation

%                             'rdp' : robot discrete perturbation

%                             'rcp' : robot continuous perturbation

%       - .perturbation.t0:   A positive scalar defining the time when the

%                             perturbation should be applied.

%       - .perturbation.tf:   A positive scalar defining the final time for

%                             the perturbations. This variable is necessary

%                             only when the type is set to 'tcp' or 'rcp'.

%       - .perturbation.dx:   A d x 1 vector defining the perturbation's

%                             magnitude. In 'tdp' and 'rdp', it simply

%                             means a relative displacement of the

%                             target/robot with the vector dx. In 'tcp' and

%                             'rcp', the target/robot starts moving with

%                             the velocity dx.

%

% Outputs ----------------------------------------------------------------

%   o x:       d x T x N matrix containing the position of N, d dimensional

%              trajectories with the length T.

%

%   o xd:      d x T x N matrix containing the velocity of N, d dimensional

%              trajectories with the length T.

%

%   o t:       1 x N vector containing the trajectories' time.

%

%   o xT:      A matrix recording the change in the target position. Useful

%              only when 'tcp' or 'tdp' perturbations applied.

%

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%%%    Copyright (c) 2010 S. Mohammad Khansari-Zadeh, LASA Lab, EPFL,   %%%

%%%          CH-1015 Lausanne, Switzerland, http://lasa.epfl.ch         %%%

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%

% The program is free for non-commercial academic use. Please contact the

% author if you are interested in using the software for commercial purposes.

% The software must not be modified or distributed without prior permission

% of the authors. Please acknowledge the authors in any academic publications

% that have made use of this code or part of it. Please use this BibTex

% reference:

%

% S. M. Khansari-Zadeh and A. Billard, "Learning Stable Non-Linear Dynamical

% Systems with Gaussian Mixture Models", IEEE Transaction on Robotics, 2011.

%

% To get latest upadate of the software please visit

%                          http://lasa.epfl.ch/khansari

%

% Please send your feedbacks or questions to:

%                           mohammad.khansari_at_epfl.ch

%% parsing inputs

if isempty(varargin)

options = check_options();

else

options = check_options(varargin{1}); % Checking the given options, and add ones are not defined.

end

d=size(x0,1); %dimension of the model

if isempty(xT)

xT = zeros(d,1);

end

if d~=size(xT,1)

disp('Error: length(x0) should be equal to length(xT)!')

x=[];xd=[];t=[];

return

end

%% setting initial values

nbSPoint=size(x0,2); %number of starting points. This enables to simulatneously run several trajectories

if isfield(options,'obstacle') && ~isempty(options.obstacle) %there is obstacle

obs_bool = true;

obs = options.obstacle;

for n=1:length(obs)

x_obs{n} = obs{n}.x0;

if ~isfield(obs{n},'extra')

obs{n}.extra.ind = 2;

obs{n}.extra.C_Amp = 0.01;

obs{n}.extra.R_Amp = 0.0;

else

if ~isfield(obs{n}.extra,'ind')

obs{n}.extra.ind = 2;

end

if ~isfield(obs{n}.extra,'C_Amp')

obs{n}.extra.C_Amp = 0.01;

end

if ~isfield(obs{n}.extra,'R_Amp')

obs{n}.extra.R_Amp = 0.0;

end

end

end

b_contour = zeros(1,nbSPoint);

else

obs_bool = false;

obs = [];

x_obs = NaN;

end

%initialization

for i=1:nbSPoint

x(:,1,i) = x0(:,i);

end

xd = zeros(size(x));

if size(xT) == size(x0)

XT = xT;

else

XT = repmat(xT,1,nbSPoint); %a matrix of target location (just to simplify computation)

end

t=0; %starting time

if options.plot %plotting options

sp = plot_results('i',[],x,xT,obs);

end

%% Simulation

i=1;

while true

%Finding xd using fn_handle.

xd(:,i,:)=reshape(fn_handle(squeeze(x(:,i,:))-XT),[d 1 nbSPoint]);

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

% This part if for the obstacle avoidance module

if obs_bool

% applying perturbation on the obstacles

for n=1:length(obs)

if isfield(obs{n},'perturbation')

if i >= round(obs{n}.perturbation.t0/options.dt)+1 && i <= round(obs{n}.perturbation.tf/options.dt) && length(obs{n}.perturbation.dx)==d

x_obs{n}(:,end+1) = x_obs{n}(:,end) + obs{n}.perturbation.dx*options.dt;

obs{n}.x0 = x_obs{n}(:,end);

xd_obs = obs{n}.perturbation.dx;

if options.plot %plotting options

plot_results('o',sp,x,xT,n,obs{n}.perturbation.dx*options.dt);

end

else

x_obs{n}(:,end+1) = x_obs{n}(:,end);

xd_obs = 0;

end

else

xd_obs = 0;

end

end

for j=1:nbSPoint

[xd(:,i,j) b_contour(j)] = obs_modulation_ellipsoid(x(:,i,j),xd(:,i,j),obs,b_contour(j),xd_obs);

end

end

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%%% Integration

x(:,i+1,:)=x(:,i,:)+xd(:,i,:)*options.dt;

t(i+1)=t(i)+options.dt;

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

% Applying perturbation if any

switch options.perturbation.type

case 'tdp' %applying target discrete perturbation

if i == round(options.perturbation.t0/options.dt)+1 && length(options.perturbation.dx)==d

xT(:,end+1) = xT(:,end) + options.perturbation.dx;

XT = repmat(xT(:,end),1,nbSPoint);

if options.plot %plotting options

plot_results('t',sp,x,xT);

end

else

xT(:,end+1) = xT(:,end);

end

case 'rdp' %applying robot discrete perturbation

if i == round(options.perturbation.t0/options.dt)+1 && length(options.perturbation.dx)==d

x(:,i+1,:) = x(:,i+1,:) + repmat(options.perturbation.dx,[1 1 nbSPoint]);

end

case 'tcp' %applying target continuous perturbation

if i >= round(options.perturbation.t0/options.dt)+1 && i <= round(options.perturbation.tf/options.dt) && length(options.perturbation.dx)==d

xT(:,end+1) = xT(:,end) + options.perturbation.dx*options.dt;

XT = repmat(xT(:,end),1,nbSPoint);

if options.plot %plotting options

plot_results('t',sp,x,xT);

end

else

xT(:,end+1) = xT(:,end);

end

case 'rcp' %applying robot continuous perturbation

if i >= round(options.perturbation.t0/options.dt)+1 && i <= round(options.perturbation.tf/options.dt) && length(options.perturbation.dx)==d

x(:,i+1,:) = x(:,i+1,:) + repmat(options.perturbation.dx,[1 1 nbSPoint])*options.dt;

end

end

% plotting the result

if options.plot

plot_results('u',sp,x,xT);

end

%Checking the convergence

if i > 3 && (all(all(all(abs(xd(:,end-3:end,:))<options.tol))) || i>options.i_max-2)

if options.plot

plot_results('f',sp,x,xT);

end

i=i+1;

%         xd(:,i,:)=reshape(fn_handle(squeeze(x(:,i,:))-XT),[d 1 nbSPoint]);

x(:,end,:) = [];

t(end) = [];

fprintf('Number of Iterations: %1.0f\n',i)

tmp='';

for j=1:d

tmp=[tmp ' %1.4f ;'];

end

tmp=tmp(2:end-2);

fprintf('Final Time: %1.2f (sec)\n',t(1,end,1))

fprintf(['Final Point: [' tmp ']\n'],squeeze(x(:,end,:)))

fprintf(['Target Position: [' tmp ']\n'],xT(:,end))

fprintf('## #####################################################\n\n\n')

if i>options.i_max-2

fprintf('Simulation stopped since it reaches the maximum number of allowed iterations i_max = %1.0f\n',i)

fprintf('Exiting without convergence!!! Increase the parameter ''options.i_max'' to handle this error.\n')

end

break

end

i=i+1;

end

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

function options = check_options(varargin)

if ~isempty(varargin)

options = varargin{1};

else

options.dt=0.02; %to create the variable

end

if ~isfield(options,'dt') % integration time step

options.dt = 0.02;

end

if ~isfield(options,'i_max') % maximum number of iterations

options.i_max = 1000;

end

if ~isfield(options,'tol') % convergence tolerance

options.tol = 0.001;

end

if ~isfield(options,'plot') % shall simulator plot the figure

options.plot = 1;

else

options.plot = options.plot > 0;

end

if ~isfield(options,'perturbation') % shall simulator plot the figure

options.perturbation.type = '';

else

if ~isfield(options.perturbation,'type') || ~isfield(options.perturbation,'t0') || ~isfield(options.perturbation,'dx') || ...

((strcmpi(options.perturbation.type,'rcp') || strcmpi(options.perturbation.type,'tcp')) && ~isfield(options.perturbation,'tf')) || ...

(~strcmpi(options.perturbation.type,'rcp') && ~strcmpi(options.perturbation.type,'tcp') && ~strcmpi(options.perturbation.type,'rdp') && ~strcmpi(options.perturbation.type,'tdp'))

disp('Invalid perturbation structure. The perturbation input is ignored!')

options.perturbation.type = '';

end

end

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

function sp = plot_results(mode,sp,x,xT,varargin)

if isempty(varargin) || isempty(varargin{1})

b_obs = false;

else

b_obs = true;

obs = varargin{1};

end

[d, ~, nbSPoint] = size(x);

switch mode

case 'i' %initializing the figure

if d==2

sp.fig = figure('name','2D Simulation of the task', 'position',[653 550 560 420]);

%screensize = get(groot, 'Screensize');

%sp.fig.Position = [0.5  * screensize(3), 0.1  * screensize(4), 0.7 * screensize(3), 0.8 * screensize(4)];

sp.axis = gca;

hold on

sp.xT = plot(xT(1),xT(2),'k*','EraseMode','none','markersize',10,'linewidth',1.5);

sp.xT_l = plot(xT(1),xT(2),'k--','EraseMode','none','linewidth',1.5);

for j=1:nbSPoint

plot(x(1,1,j),x(2,1,j),'ok','markersize',2,'linewidth',7.5)

sp.x(j)= plot(x(1,1,j),x(2,1,j),'EraseMode','none');

end

xlabel('$\xi_1$','interpreter','latex','fontsize',16);

ylabel('$\xi_2$','interpreter','latex','fontsize',16);

grid on;box on

if b_obs

[x_obs x_obs_sf] = obs_draw_ellipsoid(obs,40);

for n=1:size(x_obs,3)

sp.obs(n) = patch(x_obs(1,:,n),x_obs(2,:,n),0.1*ones(1,size(x_obs,2)),[0.6 1 0.6]);

sp.obs_sf(n) = plot(x_obs_sf(1,:,n),x_obs_sf(2,:,n),'k--','linewidth',0.5);

end

end

elseif d==3

sp.fig = figure('name','3D Simulation of the task','position',[653 550 560 420]);

%screensize = get(groot, 'Screensize');

%sp.fig.Position = [0.5  * screensize(3), 0.1  * screensize(4), 0.7 * screensize(3), 0.8 * screensize(4)];

sp.axis = gca;

hold on

sp.xT = plot3(xT(1),xT(2),xT(3),'k*','EraseMode','none','markersize',10,'linewidth',1.5);

sp.xT_l = plot3(xT(1),xT(2),xT(3),'k--','EraseMode','none','linewidth',1.5);

for j=1:nbSPoint

plot3(x(1,1,j),x(2,1,j),x(3,1,j),'ok','markersize',2,'linewidth',7.5)

sp.x(j)= plot3(x(1,1,j),x(2,1,j),x(3,1,j),'EraseMode','none');

end

if b_obs

n_theta = 15;

n_phi = 10;

x_obs = obs_draw_ellipsoid(obs,[n_theta n_phi]);

for n=1:size(x_obs,3)

sp.obs(n) = surf(reshape(x_obs(1,:,n),n_phi,n_theta), reshape(x_obs(2,:,n),n_phi,n_theta), reshape(x_obs(3,:,n),n_phi,n_theta));

set(sp.obs(n),'FaceColor',[0.6 1 0.6],'linewidth',0.1)

end

end

xlabel('$\xi_1$','interpreter','latex','fontsize',16);

ylabel('$\xi_2$','interpreter','latex','fontsize',16);

zlabel('$\xi_3$','interpreter','latex','fontsize',16);

grid on

view(-28,44)

else

sp.fig = figure('name','Simulation of the task','position',[542   146   513   807]);

for i=2:d

sp.axis(i-1)=subplot(d-1,1,i-1);

hold on

sp.xT(i-1) = plot(xT(1),xT(i),'k*','EraseMode','none','markersize',10,'linewidth',1.5);

sp.xT_l(i-1) = plot(xT(1),xT(i),'k--','EraseMode','none','linewidth',1.5);

for j=1:nbSPoint

plot(x(1,1,j),x(i,1,j),'ok','markersize',2,'linewidth',7.5);

sp.x(i-1,j)= plot(x(1,1,j),x(i,1,j),'EraseMode','none');

end

ylabel(['$\xi_' num2str(i) '$'],'interpreter','latex','fontsize',12);

grid on

if i==d

xlabel(['$\xi_' num2str(1) '$'],'interpreter','latex','fontsize',12);

end

grid on;box on

end

end

case 'u' %updating the figure

if gcf ~= sp.fig

figure(sp.fig)

end

if d==2

ax=get(sp.axis);

for j=1:nbSPoint

set(sp.x(j),'XData',x(1,:,j),'YData',x(2,:,j))

end

if max(x(1,end,:))>ax.XLim(2) || min(x(1,end,:))<ax.XLim(1) || max(x(2,end,:))>ax.YLim(2) || min(x(2,end,:))<ax.YLim(1)

axis(sp.axis,'tight');

ax=get(sp.axis);

axis(sp.axis,...

[ax.XLim(1)-(ax.XLim(2)-ax.XLim(1))/10 ax.XLim(2)+(ax.XLim(2)-ax.XLim(1))/10 ...

ax.YLim(1)-(ax.YLim(2)-ax.YLim(1))/10 ax.YLim(2)+(ax.YLim(2)-ax.YLim(1))/10]);

end

elseif d==3

ax=get(sp.axis);

for j=1:nbSPoint

set(sp.x(j),'XData',x(1,:,j),'YData',x(2,:,j),'ZData',x(3,:,j))

end

if max(x(1,end,:))>ax.XLim(2) || min(x(1,end,:))<ax.XLim(1) || max(x(2,end,:))>ax.YLim(2) || min(x(2,end,:))<ax.YLim(1) || max(x(3,end,:))>ax.ZLim(2) || min(x(3,end,:))<ax.ZLim(1)

axis(sp.axis,'tight');

ax=get(sp.axis);

axis(sp.axis,...

[ax.XLim(1)-(ax.XLim(2)-ax.XLim(1))/10 ax.XLim(2)+(ax.XLim(2)-ax.XLim(1))/10 ...

ax.YLim(1)-(ax.YLim(2)-ax.YLim(1))/10 ax.YLim(2)+(ax.YLim(2)-ax.YLim(1))/10 ...

ax.ZLim(1)-(ax.ZLim(2)-ax.ZLim(1))/10 ax.ZLim(2)+(ax.ZLim(2)-ax.ZLim(1))/10]);

end

else

for i=1:d-1

ax=get(sp.axis(i));

for j=1:nbSPoint

set(sp.x(i,j),'XData',x(1,:,j),'YData',x(i+1,:,j))

end

if max(x(1,end,:))>ax.XLim(2) || min(x(1,end,:))<ax.XLim(1) || max(x(i+1,end,:))>ax.YLim(2) || min(x(i+1,end,:))<ax.YLim(1)

axis(sp.axis(i),'tight');

ax=get(sp.axis(i));

axis(sp.axis(i),...

[ax.XLim(1)-(ax.XLim(2)-ax.XLim(1))/10 ax.XLim(2)+(ax.XLim(2)-ax.XLim(1))/10 ...

ax.YLim(1)-(ax.YLim(2)-ax.YLim(1))/10 ax.YLim(2)+(ax.YLim(2)-ax.YLim(1))/10]);

end

end

end

case 't' %updating the figure

if gcf ~= sp.fig

figure(sp.fig)

end

if d==2

ax=get(sp.axis);

set(sp.xT,'XData',xT(1,end),'YData',xT(2,end))

set(sp.xT_l,'XData',xT(1,:),'YData',xT(2,:))

if max(xT(1,end))>ax.XLim(2) || min(xT(1,end))<ax.XLim(1) || max(xT(2,end))>ax.YLim(2) || min(xT(2,end))<ax.YLim(1)

axis(sp.axis,'tight');

ax=get(sp.axis);

axis(sp.axis,...

[ax.XLim(1)-(ax.XLim(2)-ax.XLim(1))/10 ax.XLim(2)+(ax.XLim(2)-ax.XLim(1))/10 ...

ax.YLim(1)-(ax.YLim(2)-ax.YLim(1))/10 ax.YLim(2)+(ax.YLim(2)-ax.YLim(1))/10]);

end

elseif d==3

ax=get(sp.axis);

set(sp.xT,'XData',xT(1,end),'YData',xT(2,end),'ZData',xT(3,end))

set(sp.xT_l,'XData',xT(1,:),'YData',xT(2,:),'ZData',xT(3,:))

if max(xT(1,end))>ax.XLim(2) || min(xT(1,end))<ax.XLim(1) || max(xT(2,end))>ax.YLim(2) || min(xT(2,end))<ax.YLim(1) || max(xT(3,end))>ax.ZLim(2) || min(xT(3,end))<ax.ZLim(1)

axis(sp.axis,'tight');

ax=get(sp.axis);

axis(sp.axis,...

[ax.XLim(1)-(ax.XLim(2)-ax.XLim(1))/10 ax.XLim(2)+(ax.XLim(2)-ax.XLim(1))/10 ...

ax.YLim(1)-(ax.YLim(2)-ax.YLim(1))/10 ax.YLim(2)+(ax.YLim(2)-ax.YLim(1))/10 ...

ax.ZLim(1)-(ax.ZLim(2)-ax.ZLim(1))/10 ax.ZLim(2)+(ax.ZLim(2)-ax.ZLim(1))/10]);

end

else

for i=1:d-1

ax=get(sp.axis(i));

set(sp.xT(i),'XData',xT(1,end),'YData',xT(i+1,end))

set(sp.xT_l(i),'XData',xT(1,:),'YData',xT(i+1,:))

if max(xT(1,end))>ax.XLim(2) || min(xT(1,end))<ax.XLim(1) || max(xT(i+1,end))>ax.YLim(2) || min(xT(i+1,end))<ax.YLim(1)

axis(sp.axis(i),'tight');

ax=get(sp.axis(i));

axis(sp.axis(i),...

[ax.XLim(1)-(ax.XLim(2)-ax.XLim(1))/10 ax.XLim(2)+(ax.XLim(2)-ax.XLim(1))/10 ...

ax.YLim(1)-(ax.YLim(2)-ax.YLim(1))/10 ax.YLim(2)+(ax.YLim(2)-ax.YLim(1))/10]);

end

end

end

case 'o' %updating the obstacle position

if gcf ~= sp.fig

figure(sp.fig)

end

if b_obs

n = varargin{1};

dx = varargin{2};

if d==2

set(sp.obs(n),'XData',get(sp.obs(n),'XData')+ dx(1))

set(sp.obs(n),'YData',get(sp.obs(n),'YData')+ dx(2))

set(sp.obs_sf(n),'XData',get(sp.obs_sf(n),'XData')+ dx(1))

set(sp.obs_sf(n),'YData',get(sp.obs_sf(n),'YData')+ dx(2))

elseif d==3

set(sp.obs(n),'XData',get(sp.obs(n),'XData')+ dx(1))

set(sp.obs(n),'YData',get(sp.obs(n),'YData')+ dx(2))

set(sp.obs(n),'ZData',get(sp.obs(n),'ZData')+ dx(2))

end

end

case 'f' % final alighnment of axis

if gcf ~= sp.fig

figure(sp.fig)

end

if d==2

axis(sp.axis,'tight');

ax=get(sp.axis);

axis(sp.axis,...

[ax.XLim(1)-(ax.XLim(2)-ax.XLim(1))/10 ax.XLim(2)+(ax.XLim(2)-ax.XLim(1))/10 ...

ax.YLim(1)-(ax.YLim(2)-ax.YLim(1))/10 ax.YLim(2)+(ax.YLim(2)-ax.YLim(1))/10]);

elseif d==3

axis(sp.axis,'tight');

ax=get(sp.axis);

axis(sp.axis,...

[ax.XLim(1)-(ax.XLim(2)-ax.XLim(1))/10 ax.XLim(2)+(ax.XLim(2)-ax.XLim(1))/10 ...

ax.YLim(1)-(ax.YLim(2)-ax.YLim(1))/10 ax.YLim(2)+(ax.YLim(2)-ax.YLim(1))/10 ...

ax.ZLim(1)-(ax.ZLim(2)-ax.ZLim(1))/10 ax.ZLim(2)+(ax.ZLim(2)-ax.ZLim(1))/10]);

else

for i=1:d-1

axis(sp.axis(i),'tight');

ax=get(sp.axis(i));

axis(sp.axis(i),...

[ax.XLim(1)-(ax.XLim(2)-ax.XLim(1))/10 ax.XLim(2)+(ax.XLim(2)-ax.XLim(1))/10 ...

ax.YLim(1)-(ax.YLim(2)-ax.YLim(1))/10 ax.YLim(2)+(ax.YLim(2)-ax.YLim(1))/10]);

end

end

end

drawnow